# Study on Emergency Linkage Non-conflicting Optimal Path for Urban Grave Disaster

1. YANG Zhen-hong  2. LEI Ying  3. HE Juan-xia

1. School of Energy Engineering, Xi’an University of Science & Technology, Xi’an, P.R. China, 710054
1.2. School of Material Science and Engineering, Xi’an University of Architecture & Technology, 
Xi’an, P.R. China, 710055
3. Colleges of Resources and Environment, Guangxi University, Nanning, P.R. China, 530004

Abstract  City is a complex dynamic system which is composed by principal parts such as people and matter, by substantiality such as resource, economy, and circumstance, where the subsystems are interdependence, interaction and relative restraint. The establishment of Urban Grave Disaster Emergency Rescue System is an important guarantee of urban safety development and social stability. Based on the theory of Shortest and Optimal Path and the powerful programming function of LINGO, with the basic requirements of the wrecking car to select paths (such as speed, quantity of containing vehicle, etc.) under consideration, building an emergency model with non-conflicting linkage optimal path for urban grave disaster, which applies the LINGO programme for account and optimization(two circs: (1)take the authority power value influencing the choice of the optimal path into account; (2) don’t take the authority power value influencing the choice of the optimal path into account), so as to send emergency materials to the accident spot as quickly as possible and to reduce the accident loss.

Key words  Disaster emergency rescue, Optimal path, Lingo language, Authority value

## 1 Introduction

Recently, all sorts of urban grave disasters including the terrorism, have become an enormous threat to urban public safety, the governments and scientists all over the world have conducted a comprehensive exploration and efforts for preventing urban disasters in protecting of society stability and property safety of people’s life, However, people also seemed to be helpless when they are facing to an unexpected grave disaster in the city. There are a vast number of personnel deaths with huge economic losses which were caused by all kind of grave disasters in this day and age [1]. According to the national principle which is set to prevent the emergency rescue system of grave disaster and one way is brought out that is said as “put focus on the main city in a province and make its influence radiating through to the towns around” , this method is to practice to take the capital as center in each province, covering with the network unit of other cities, Sequentially, becoming an integrative reducing and preventing disaster system including four grades of nations, provinces, cities and counties, in order to strengthen the ability of reducing and preventing the disasters [2].

Type of two grades linkage: Taking Beijing as a representative. Set up the grave disaster emergency rescue system in the city and area, and it is taken the charge completely by mayor. The system integrates the public security system (110,122), fire fight system (119), emergency medical treatment system (120), public hygiene, natural disaster and the city lifeline and other resources together, which is becoming the two grades linkage system, and extends to the county and grass-roots community.

Type of the whole linkage: Taking Xi’an as a representative. Set up the grave disaster emergency rescue system in the province, cities, local towns, villages...etc, and it is taken charge completely by mayor. The system integrates the public security system (110,122), firefight system (119), emergency medical treatment system (120) and other resources together, which is becoming the whole linkage center, covered with the grass-roots community [3].

Because of the differences of economic development and restrictions of geographical location in our country, each province is under the premising condition that is the rational distribution and unified planning of the grave disaster emergency rescue system, which is the most important factor deciding whether they can deliver the various rescue supplies to the grave disaster place and to send the injured to a hospital in the shortest time. This paper combines the present condition of a certain city transportation network, when occurred the grave disaster, how to choosing the optimal emergency rescue path quickly, using the LINGO software to have a optimal calculation, so as to realize the request of emergency linkage for non-conflicting dynamic control: ①Ordinarily, the system of the emergency linkage estate is gradual and adaptable to the optimal control goal(each organization operate with their own plan goal, allow to appear the phenomenon of furcation, composite and conflict);②During training, the system of the emergency linkage estate is sforzando and adaptable to the optimal control goal( each organization operate with their bailout plan goal , do not allow to appear the phenomenon of furcation, composite and conflict); ③During the war, the system of the emergency linkage estate is tracing and adaptable to the optimal control goal(each organization operate with their bailout plan goal, do not allow to appear the phenomenon of furcation, composite and conflict) [4] [5].

## 2 The model of optimal path

### 2.1 Establish the optimal path mathematics model

Using the city transportation network as the foundation to build up the accident rescue optimal path model [6], defining each crossing as a node and between the two nodes is a arc, giving a certain network G(N, A), the node set N and the arc set A, and define:

(1) Emergency rescue set L, point i∈L, can provide ri rescue vehicles;

(2) Accident point set F(|F|=m), point f∈F needs nf rescue vehicles;

(3) λiv expresses the corresponding value from point i to point v (i∈L，v∈N)in the network G (this value is considered as a synthetic index, and the value that the λiv is the optimal path summation which is from point i to point v , it means the value is corresponding sections summation of the optimal path. Assumptions the optimal path which is from point i to point v passing the point wn，wn∈N，n=1,2…,using βwn to express the absolute path which is between the two corresponding nodes, each corresponding value of the absolute path is λ│βwn, obtained λiv=∑λ│βwn).

(4)

(5) xif expresses the number of the rescue vehicles which is from the point i∈L to the point f∈F .

Each node in the network needs only one rescue vehicle when the recurrence accident occurs (on the space S.D.Poisson assumption that implied the synchronization of the recurrence accident). However, the actual circumstance is more complicated, dealing with more and more rescue vehicles and recurrence accident, which need to improve the model, so that to meeting more and more complicated circumstances.

For solving a more complicated circumstance which is an actual accident emergency rescue (the more accident points, the more rescue vehicles for respond), due to the shortage of the general model, through further analysis to building up a new accident rescue model (MIMR-MODEL) descript as follows:

(1)

Be restricted by the following conditions:

(2)

(3)

(4)

(5)

(6)

In the formula:

(1) Analyzed an actual circumstance, the rescue point in the rescue model (1) should be limited, and it can increase the rescue point from the others if a rescue point of resources is not enough in a certain of the accident point; Actually, value λ which is applied in the system is not just the factor of distance and so on, but should consider the other related factors.

(2) The type (2)-described the number of the rescue vehicles which is sent from the point i∈L to a certain of an accident point f∈F shouldn’t exceed to the total numbers of the rescue vehicles which is in the rescue point i∈L, That is to say: when the system makes a policy, ensure the possibility of the system and couldn’t overdraw resources of a certain rescue point; si expresses that number of the surplus rescue vehicles for the recurrence accident which is sent a certain quantity from the rescue point i∈L to f∈F, namely the surplus resources of the rescue point in the city can be send to assurance the rescue resources were made its best in the system.

(3) The type (3)-ordered that summation of the number of the rescue vehicles which is sent from the rescue point should equal to the number of the rescue vehicles which is the accident point needed, namely the rescue resources should meet the need of the rescue, it is one of the sign which is made the best of the resources.

(4) The type (4)-according to the definition of yiv ,and the rescue vehicles in rescue point are ready to go at any time for the recurrence accident rescue response, as long as rescue vehicles for the rescue response, variable yiv in the model become1.

(5) The type (5)-stipulated the surplus vehicles can be used which is under this precondition that a certain points of rescue vehicles can join in the rescue in the type (1).

(6) The type (6)-limited the variable y can only choose 0 or 1, choosing the rescue point 0 represent that those resources can’t be operated, and vice versa; variable x can only choose the bigger than integer 0 (that is to say the number of vehicles only a integer).

### 2.2 Optimal path to achieve the model algorithm

Model (1) as a network analysis platform of design algorithm, though the network analysis of drawing put the geographic reality of the network to achieve the optimization issue of geographical network. The optimal path of the analysis in the network is based on the analysis of the shortest path, so the analysis of the shortest path issue is the most basic and most crucial, and the solution to analysis methods of the shortest path have been very matured, recently, using the theory basis Dijkstra which is the most classic and most common to solving the issue of the shortest path.

## 3 Lingo achieve optimal path decision-making programme

### 3.1 Construction of the optimal path selection

When a work accident occurs, even if when using an advanced computer simulation software, it is also impossible to accurately estimate the developments in the accident, no one can guarantee that the development of the situation would not cause a huge casualties and property losses, so when sufficient consider a work accident occurred, the accident should be timely controlled, and to prevent the expansion of the situation, it is become an crucial issue to determine whether the emergency supplies could be transported to the accident site in the shortest time [7].

In normal circumstances, which is related to an emergency rescue of work accidents, according to the distinctness of the nature of an accident and the size of the criticality which occurred after the accident, considering the safety and the time of the transport to choose the optimal path of emergency rescue. In this paper, in order to predigest the model, obtaining the shortest path quickly, only to consider the influence of the speed and capacity, and transport the emergency supplies to the accident site in a shortest time. When considering the influence of the speed and capacity to choose the optimal path, the optimal path, which is calculated, may not be the actual shortest distance, but it is the optimal path, which can transport the emergency supplies to the accident site in a shortest time.

Considering the negative cases of the path bij＞0 in the model, the mathematical model can be summarized as follows:

Assuming that the total numbers of the cities are M, the another city would transport materials to which occurred a work accident, these M cities was expressed with Pi(1,2,…,M)and composed a set {Pi},the distance which is from a point Pi to another point Pj was expressed with bij, if there isn’t a path linkage from a point Pi to another point Pj ,the bij was stipulated as bij=+∞，in this paper stipulated the bij=0(1≤i≤M) ,and designated an end-point PM , demanding the shortest path which is from the city Pi to another city PM. Using the method of dynamic programming to solve this problem in this paper, Using the city Pi to express the state, the set of decision-making is a city except the city Pi, after having selected a city Pj, gained the distance bij and get it into a new state, that is, another city Pj, When the state is the city PM, stop to do the process, during the process which is to choice the path, should be considered the chosen path grade. From the above analysis can be seen, this is aperiodicity and have more phases decision-making process.

Therefore, when the accident occurred, taking full account of the above-mentioned conditions, assumptions the direction is from P1 to PM:

(1) the direction of each arc (URL) is former;

(2) the capacity of each arc (URL) is more than the others;

(3)the capacity of each arc (URL) can be neglected which is Continued to increase, and can accommodate the capacity of the previous arc (URL).

Get the model of the optimal path, the model formula such as formula (7) below:

f(i)=minjaijbij+f(j),i=1,2,…,M−1               (7)

In the formula：the influence of authority value aij is for the road grading to selected path, as shown in Table 1; bij is the length of each road, according to the actual situation to choose; m is the number of the city which for the supply of materials urgent interim，i∈m;

Table 1  The chosen path impact of work accident emergency rescue in the highway grading

| Influencing Factors Highway grade | Speed | Traffic capacity | Value (aij) |
| --- | --- | --- | --- |
| Thruway | 120km/h~80km/h | A variety of cars converted into small passenger cars in the average of day and night during a year, the number of the traffic capacity is more than 25,000 | 1~0.9 |
| Highway 1 | 80km/h | A variety of cars converted into small passenger cars in the average of day and night during a year, the number of the traffic capacity between 15000~30000 | 0.9~0.8 |
| Highway 2 | 80km/h~60km/h | A variety of cars converted into medium-sized camion in the average of day and night during a year, the number of the traffic capacity between3000~7500 | 0.8~0.6 |
| Highway 3 | 40km/h~30km/h | A variety of cars converted into medium-sized camion in the average of day and night during a year, the number of the traffic capacity between 1000~4000 | 0.6~0.5 |
| Highway 4 | 20km/h | A variety of cars converted into medium-sized camion in the average of day and night during a year, the number of the traffic capacity is less than 1500 in two-lane, and in single carriageway there is less than 2010 | 0.5~0.3 |

### 3.2 Lingo software application

The United States LINGO Systems (Lingo System Inc.) developed the LINGO software package, for solving the issues of large-scale mathematical programming, it could solve the issues of linear programming, integer programming problems, quadratic linear and non-linear planning and other issues, as well as the graph theory and the network combinatorial optimization problem.

It is an essential basic work of using the LINGO software to realize the optimal path analysis system in the accident emergency rescue, and the core of the optimal path analysis is the efficient implementation of the shortest path algorithm. Because of optimal path analysis system was widely applied foreground in the production of these major accident emergency system, which is have a high real-time requirements for the optimal path analysis [8]. When the accident occurred, after determining the alarm points and the scene of the accident, it is one of the key issues that the city work accident emergency rescue centre how to decided to take the succor material to the place of accident quickly and effectively in the shortest time in the system, the high efficiency implementation of materials transportation can improve the capability of the rapid reaction and integer direct combat. Therefore the effective implementation of the optimal path algorithm directly determined the optimal path background, in this discussion of the optimal path is mainly to the authority value of all arcs are the plus plane chart and research how to find out the optimal path between the two nodes quickly [9] [10].

## 4 Analysis the examples of determinate emergency rescue optimal path

The calculate realized in the analysis of the shortest path model and LINGO software, through the following examples to analysis, as shown in Figure 1, that is, when node 10 occurred the work accident, needing the emergency supplies transport from node 1 to node 10, and calculating the optimal path which transport the emergency supplies from the transporting supplies node 1 to the accident node 10 (various parameters shown in Table 2). To be taken into account the speed of the traffic and the capacity (value aij) in calculating which is on the impact of choosing the optimal path.

Figure 1  Schematic diagram of choice shortest path

Table 2  The corresponding parameters of nodes in the all various path

| Parameters Sections | aij | 1- aij | bij(km) | Parameters Sections | aij | 1- aij | bij(km) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Node1- Node2 | 0.8 | 0.2 | 48 | Node4- Node8 | 0.4 | 0.6 | 8 |
| Node1- Node3 | 0.85 | 0.15 | 40 | Node5- Node7 | 0.7 | 0.3 | 64 |
| Node2- Node4 | 0.7 | 0.3 | 24 | Node5- Node8 | 0.7 | 0.3 | 56 |
| Node2- Node5 | 0.65 | 0.35 | 48 | Node5- Node9 | 0.45 | 0.55 | 40 |
| Node2- Node6 | 0.55 | 0.45 | 72 | Node6- Node8 | 0.7 | 0.3 | 32 |
| Node3- Node4 | 0.85 | 0.15 | 56 | Node6- Node9 | 0.4 | 0.6 | 10 |
| Node3- Node5 | 0.7 | 0.3 | 40 | Node7- Node10 | 0.85 | 0.15 | 40 |
| Node3- Node6 | 0.55 | 0.45 | 88 | Node8- Node10 | 0.7 | 0.3 | 56 |
| Node4- Node7 | 0.95 | 0.05 | 72 | Node9- Node10 | 0.65 | 0.35 | 72 |

### 4.1 The calculation of the optimal path without authority value

Adopting the model (1) on this issue, through the programming, when inputting the calculator and obtaining the optimal path of transport supplies from node 1 to node 10. Table 3 is calculated results of LINGO. F(i ) expressed that the shortest path from node 1 to node 10, when the value of P(i，j)(i≤10，j≤10) is 0, indicated that don’t choose the path of transporting the emergency succor supplies which is from node i to node j, when the value of P(i，j)(i≤10，j≤10) is 1, indicated that choose the path of transporting the emergency succor supplies which is from node i to node j. Therefore, the optimal path of transporting the emergency succor supplies is from the node 1, node 2, node 4, node 8 to node 10, the actual total transportation distance is 136km, and the non-conflicting linkage distance is 38.4 km. Table 3 shows.

Table 3  The calculation result of the optimal path without authority value

| Feasible solution found at iteration: |  |  |  |  | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Value | Variable | Value | Variable |  | Value | Variable | Value |
| M | 10.00000 | B(1,2) | 48.00000 | B(5,9) |  | 40.00000 | P(3,5) | 0.000000 |
| F(1) | 136.0000 | B(1,3) | 40.00000 | B(6,8) |  | 32.00000 | P(3,6) | 0.000000 |
| F(2) | 88.00000 | B(2,4) | 24.00000 | B(6,9) |  | 10.00000 | P(4,7) | 0.000000 |
| F(3) | 120.0000 | B(2,5) | 48.00000 | B(7,10) |  | 40.00000 | P(4,8) | 1.000000 |
| F(4) | 64.00000 | B(2,6) | 72.00000 | B(8,10) |  | 56.00000 | P(5,7) | 1.000000 |
| F(5) | 104.0000 | B(3,4) | 56.00000 | B(9,10) |  | 72.00000 | P(5,8) | 0.000000 |
| F(6) | 82.00000 | B(3,5) | 40.00000 | P(1,2) |  | 1.000000 | P(5,9) | 0.000000 |
| F(7) | 40.00000 | B(3,6) | 88.00000 | P(1,3) |  | 0.000000 | P(6,8) | 0.000000 |
| F(8) | 56.00000 | B(4,7) | 72.00000 | P(2,4) |  | 1.000000 | P(6,9) | 1.000000 |
| F(9) | 72.00000 | B(4,8) | 8.000000 | P(2,5) |  | 0.000000 | P(7,10) | 1.000000 |
| F(10) | 0.000000 | B(5,7) | 64.00000 | P(2,6) |  | 0.000000 | P(8,10) | 1.000000 |
|  |  | B(5,8) | 56.00000 | P(3,4) |  | 1.000000 | P(9,10) | 1.000000 |

In the calculation, if doesn’t take into account in the traffic levels of the chosen path (speed and traffic volume), and then using LINGO to predigest the calculated steps, so as to prevent errors in calculation. But，if the calculation of the optimal path without authority value ,that is because they does not take into account the influence on chosen path for the transport of emergency supplies, so there is likely to occurred the work accident, blinding to choose the path of emergency supplies which seems reasonable, and hoping that transport the emergency rescue resources to the accident site in the shortest time, but sometimes may be a counterproductive behavior, and leading to an unnecessary losses.

### 4.2 The calculation of the optimal path with authority value

Selecting the transport path of emergency supplies, it should be considered that the path in actual situation at that time, for example, the carrying capacity of the road, the traffic flow size of the path, the highway grade, the speed of traffic, the transport capacity of the machine tools about emergency goods and so on , among these many influence factors of transport material, speed and traffic flow is the most of the important factors during the process of transport emergency supplies, so, when calculating the path, should be taken into account the influence of speed and traffic volume to choose the path of transport emergency supplies.

In Table 3, there are the same analysis between F(i), P(i，j)(i≤10，j≤10) and the shortest-path without the authority value ,among those, the Bij[(1-aij)•bij] expressed “the path” that when considering the influence of speed and traffic volume . Therefore, through the analysis calculation of LINGO software, obtaining that optimal path which is from the node 1, node3, node4, node7 to node 10 of transporting the emergency succor supplies is from the initial place (node 1) to the accident took place (node 10), the actual total transportation distance is 208 km, and the non-conflicting linkage distance is 24 km. Table 4 shows.

Table 4  The calculation results of the optimal path with affixation authority value

| Feasible solution found at iteration: |  |  |  | 0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Variable | Value | Variable | Value | Variable | Value | Variable | Value |
| M | 10.00000 | B(1,2) | 9.600000 | B(5,9) | 22.00000 | P(3,5) | 0.000000 |
| F(1) | 24.00000 | B(1,3) | 6.000000 | B(6,8) | 9.600000 | P(3,6) | 0.000000 |
| F(2) | 16.80000 | B(2,4) | 7.200000 | B(6,9) | 6.000000 | P(4,7) | 1.000000 |
| F(3) | 18.00000 | B(2,5) | 16.80000 | B(7,10) | 6.000000 | P(4,8) | 0.000000 |
| F(4) | 9.600000 | B(2,6) | 32.40000 | B(8,10) | 16.80000 | P(5,7) | 1.000000 |
| F(5) | 25.20000 | B(3,4) | 8.400000 | B(9,10) | 25.20000 | P(5,8) | 0.000000 |
| F(6) | 26.40000 | B(3,5) | 12.00000 | P(1,2) | 0.000000 | P(5,9) | 0.000000 |
| F(7) | 6.000000 | B(3,6) | 39.60000 | P(1,3) | 1.000000 | P(6,8) | 1.000000 |
| F(8) | 16.80000 | B(4,7) | 3.600000 | P(2,4) | 1.000000 | P(6,9) | 0.000000 |
| F(9) | 25.20000 | B(4,8) | 4.800000 | P(2,5) | 0.000000 | P(7,10) | 1.000000 |
| F(10) | 0.000000 | B(5,7) | 19.20000 | P(2,6) | 0.000000 | P(8,10) | 1.000000 |
|  |  | B(5,8) | 16.80000 | P(3,4) | 1.000000 | P(9,10) | 1.000000 |

## 5 Conclusion

From the calculated results without the authority value of the optimal path can be seen, in the case of work accident, the optimal path A which the supplies node1 transport the emergency supplies to the accident node 10 is from the node1, node 2, node 4, node 8 to node10, the actual total transportation distance is136 km, taking into account the speed and capacity, the non-conflicting linkage distance equivalent to 38.4 km；and while considering the influence of the Authority Value to transport the emergency supplies, and get the optimal path B is from the node 1, Node 3, node 4, node 7 to node 10, the actual total transportation distance is 208 km, taking into account the speed and capacity, the non-conflicting linkage distance equivalent to 24 km, obviously, the optimal path B is better than A.

##### References

[1]	Feng Changgen. A Review of Accidents and Disasters in China in 2003[J]. Journal of Safety and Environment, 2005, (5)1:1-8 (in Chinese)

[2]	Niu Xiaoxia,et.al. Study on the Mode of Urban Public Safety Planning [J].China Safety Science Journal .2003,13(10): 1-3 (in Chinese)

[3]	Liu Tiemin,et.al. Probe into Response at Different Level and Rescue Procedure of National Emergency Response System for Work Safety [J]. China Safety Science Journal .2003, 13(12): 5-8 (in Chinese)

[4]	Xu Zhisheng,et.al. Experimental Scheme of Urban Public Security for Changsha city [J]. China Safety Science Journal .2005, 15(1): 3-5 (in Chinese)

[5]	Gu Linsheng, et.al. Prevent the Disaster Synthetically and Crisis Management for Urban [J].China Public Security. 2005(2):41-46 (in Chinese)

[6]	Chen Geng, Liu Mao, et.al. Road Traffic Risk Analysis and Its Application in Tianjin city[J].China Safety Science Journal .2006,16(5):52-55 (in Chinese)

[7]	Lu Zhaoming, et.al. The City Urgent Evacuate System Based on GIS [J]. China Public Security. 2005, (5) 1:1-8 (in Chinese)

[8]	Yu Hanhua, et.al. Distribution Rationality of Great Hazard Resources in Cities and the Sustainable Development [J]. Industrial Safety and Dust Control. 2005,31(1): 35-37 (in Chinese)

[9]	Yang Zhenhong, et.al. An Expert System for Risk Assessment Tools of Construction Projects [C].2002 ISSST, 2002, 10:1013-1016

[10]	Yang Zhenhong, et.al. Studies on the Strategy and Technology of Risk Foresight on Urban Public Security [C]. 2004 ISSST, 2004, 9. 2928-2932