# Evaluation of Sustainable Development of 
Urban Rail Transit Project

1.LIU Lin  2.CHEN Ji-hua

1.2. College of Environmental Science and Engineering, Donghua University, P.R. China, 201620

1.School of Business, Zhejiang Textile and Fashion College, P.R. China, 315211

Abstract  It is a crucial time for the rapid development of urban rail transit construction, and much attention should be paid to the strategic evaluation in the sustainable development field. In order to ensure the healthy development of the urban rail transit, it is necessary to constitute the supporting measures. Aiming at the main factors restricting the sustainable development of the urban rail transit, a series of indices were picked up and the evaluating method was established which is based on the grey relational analysis (GRA). With the help of this method, the best solution is selected among four rail transit routes. The result shows that the sustainable development evaluation method, which is based on GRA, is simple and scientific, and has a promising prospect.

Key words  Urban rail transit, Public participation, Grey relational analysis, Sustainable development

## 1 Introduction

There are a series of issues facing public transit, such as transit congestion, parking cost, and traffic accidents. The urban rail transit is one of the most effective transportation methods. However, the urban rail transit project takes a long time to finish, has significant influence on the urban economy and its construction is not reversible [1-2]. Therefore, it is very important to evaluate the built urban rail transit projects and find the best solution to provide references to other similar projects.

There are many factors influencing on the urban rail transit projects including internal factors and external factors. The former include decision making, design, construction and operation et al. The latter include environment protection, public participation, macro economy and consumer utilities et al [3-5]. It is difficult to describe the development trend of the system with traditional evaluation methods when the solutions involve the interference of quantifiable and qualitative indices. However, the grey relational analysis (GRA) can help analyze related factors, describe the development trend of the system and provide scientific methods for the decision making of the large construction projects.

Grey system theory was formulated by Ju-long Deng in 1982[6]. According to this theory, a system whose internal information—such as architecture, operation mechanism, system characteristics and parameters—are completely known is called a white system. On the contrary, a system is defined as a black system if one cannot obtain any information and characteristics about the system. Grey system is thus defined as a system defined between the white and black systems. So the information and messages of the grey system are partially clear, but some are not. GRA is based on the grey theory. Compared with the traditional regression statistical methods, which require more data and time, GRA needs less data to get reasonable and good results. In addition, GRA has been applied in many fields, such as industry, energy sources, transit, economics etc [7-10].

In this paper, the sustainable development of the urban rail transit project was evaluated under GRA from another point of view.

## 2 The sustainable development of the urban rail transit

### 2.1 The present situation of urban rail transit project in China

The urban rail transit system mainly includes the subway and light rail. It has many advantages such as resource saving, high passenger carrying ability, short intervals, punctuality and the direct link between downtown and suburb. Therefore it has been adopted widely by more and more countries. As for the subway, it has existed for approximately one century. Many international metropolises have built hundreds of kilometers of city transportation network such as New York, Tokyo, London, Paris and Moscow. The subway has become the major transportation vehicle for the city settlers.

Comparing with the western cities, the urban rail projects in China started later and the density effects and dimensionality effects of rail transit are at a low level. The people using subway or light rail accounts for only a small percentage, which is smaller than that of western cities.

Since “the 11th-five-year plan”, the urban rail construction in large cities in China has greatly developed. Beijing, Shanghai and Guangzhou are the earliest cities to construct the rail transit projects. Then Shenzhen, Chongqing, Wuhan, Nanjing, Tianjin, Chengdu, Shenyang, Hangzhou, Qingdao, Xi’an, Suzhou, Ningbo and others followed closely. Some of these cities have built a section of urban rail and others have started the experimental work or made line-drawings.

### 2.2 The meaning of the sustainable development evaluation of the urban rail transit

When Norway Prime Minister, Gro Harlem Brundtland, was the chairman of the United Nations Committee on Environment and Development in 1987, she gave her report “Our Common Future”, in which she defined the “sustainable development” as satisfying present needs without compromising the ability of future generations to meet their needs. This definition was commonly accepted by the United Nations Conference on Environment and Development in 1992. In order to gain sustainable development, it is necessary to solve these key problems, such as population, resource, environment and development, which are shorted as PRED problems. Two main parts are involved in the sustainable development, which are needs and restriction on needs. The former means to satisfy the basic needs of the poor firstly. The latter refers to the restriction on the compromising the future environment. Once the compromising is there, it will surely endanger the ecological system, such as atmosphere, water, soil and the living things.

The sustainable development of a society has laid a sound theoretical basis for the sustainable development of the urban rail transit, but there are very few studies in this field in China. The existing investigations are mainly concentrated on the methods to realize the sustainable development [11]. According to the overall requirements of building an economical society, and in order to fulfill the scientific approach from an all-round way, it is necessary to carry out a scientific evaluation on the sustainable development of the urban rail transit, to conclude the previous experience and failures so as to provide workable strategic references for the future projects.

The research conducted by the World Bank shows that the sustainability of the urban rail transit consists of three aspects [12]. One is the sustainability of economy and finance. It requires the effective usage of the related resources. Another is the sustainability of the environment and ecology. It needs to consider all the negative effects brought by transportation. The last one is the sustainability of the society. It is asked to reduce traffic congestion and benefit the public. Therefore, the sustainable development of urban rail transit can be interpreted as the rationality of the economic benefits, the compatibility of environment and ecology, as well as the acceptability of the public.

### 2.3 The significant factors influencing the sustainable development of urban rail transit

The urban rail transit involves many links due to its long existing period. Therefore, various factors, such as conceptual, economic, planning and equipment ones can deeply influence its sustainability.

2.3.1 Conceptual factors

At present, the construction of urban rail transit is unprecedented. But it is the insufficient examples to follow and purposely shortening the time limit of the project that cause the line planning unreasonable; cause the construction expense high and the unsolved problems even harder to settle. As to the size of the station, different cities show their ambitions in building a much bigger one, with two stories or even three. The luxurious decoration and the safety problems not only increase the cost of the project, but also enhance the consumption of electricity.

2.3.2 Economic factors

The investment to urban rail transit is huge, usually up to tens of billions RMB. The investment to civil facilities is a deposit, which can not produce the economic benefits directly, and the extremely high import rate of electronic-mechanical equipments will take up much of the reset cost. With the ticket earning alone, we can not afford the regular expenses, saying not the interest and depreciation of the previous investment.

However, the rail transit has tremendous social benefits. It can help reduce the traffic congestion and resource consumption, and to improve the utility rate of the urban land. The people throughout the country are concerning about the problems, such as how to coordinate the contradiction between the social benefit and that of the economy, how to internalize the out benefits, and how to avoid adding burden to the regional finance [13].

In order to unite economy benefit and social benefit, the rail transit ticket should be lowly priced. Because it’s necessary to consider the low-income families and the needs of the underprivileged group and constitute the all-round development strategy.

2.3.3 Planning factors

At present, the planning of urban rail transit in China is still at the primary stage [14]. It is often out of joint with the overall urban planning, and frequently changed. So it is not authoritive. Its stations are not near enough with those of other vehicles. The underground space is seriously spoiled. These two disadvantages directly restrict the economic benefit of the rail transit network. Let’s take the R1-line Xujiahui station in Shanghai for example, when it was first planned, there was no construction land for further establishment, thus caused great trouble to the construction of R9-line, and also lengthen the distances among the stations.

2.3.4 Equipment factors

The urban rail transit involves many sectors, such as project machinery, locomotive vehicle, tractive power supply, communication signals, automatic controlling, aeration-conditioner, safety and disaster-preventing and so on[15]. The capacity of homebred equipment falls far behind that of the world famous brand. For instance, the train automatic controlling system is owned mainly by Alstom, Simens, Bombardier, and others transnational companies. In China, the equipment of different lines and even the stations on the same lines are from different suppliers. The leading result is that the electronic-mechanical equipments are not in accordance with the systematic patterns; that the capacity is poor; that management is not easy. To some extent, these results not only increased the difficulty to renew the existing lines, but also block the process of making the rail transit equipments domestic.

## 3 The establishment of the evaluation index system of the sustainable development

The evaluation of sustainable development of the urban rail transit is trans-discipline-featured. It involves the Management Science, Economics, Engineering Science, Environment Science, Sociology and so on. Therefore, it is necessary to avail the points of system theory to integrate the evaluation, whose indices are, in fact, established on the basis of the above disciplines.

### 3.1 The establishment principles of the evaluation index system

3.1.1 Systematic principle

The urban rail transit project is a systematic project which involves multiple disciplines and fields. It should be viewed from the points of system theory by identifying the urban rail transit and specific functionalities, the harmonious development of regional economy and macro economy and the protection of the environment and historical sites.

3.1.2 The scientific principle

The scientific principle is the fundamental principle for building the index system. The indices should be identified explicitly. The index system should be based on the sustainable development theory and focus on the long term influence on the city future development.

3.1.3 The principle of the selection criteria

The urban rail transit project is a linear and point to point interconnected system. It is restricted by many conditions such as the area economy development level, the supporting infrastructure, related laws and rules et al. It is important to choose the important criteria to reflect the main issues of the system.

## 3.2 The composition of the index system

After consulting to experts in the rail transit and related fields, eight indices affecting the sustainable development are chose based on the above establishment principles [16-18].

3.2.1 The rationality of the consulting planning

The main tasks of consulting planning are as follows: to form the project company which contains people who major in the technology, economy, law, politics and other related fields; to design the plan of rail transit lines and network; to make the construction plan on rail transit; to compile the research report on investment chances and the feasibility; to forecast and estimate the risk; to budget and submit the plan on financing; to make project design; to complete dismantling and allocation and to invite public bidding. The consulting planning of the urban rail transit project is the most important part and its rationality directly affects the sustainable development of the urban rail transit.

3.2.2 The scientific reasoning of the overall design

The overall design mainly includes network design and construction design. The former covers the following parts: the network architecture, the direction of routes, the length of lines, the construction scope, the time order of the construction; The latter includes the deep of the lines, the position of the stations, the earth control, the position of the vehicles, the supply of the power, the arrangement of landscape, the public safety projects, the protection of culture relics, the environmental evaluation and the rationality of fund raising plan.

3.2.3 The coordination of the construction and equipment installation

This includes the contract management of the project, the risk control, the control of quality, schedule and price of engineering, the security support, the environment protect, the changes of project, the fund support and the check activities after the project is done. At the construction stage, it is necessary to pay attention to surface subsidence, dust and residue, wastewater discharge, construction noise, transit congestion, and underground pipeline transfer.

3.2.4 The consistent operation management

The focuses of this job is to analyze the mode and effects of operation management, which affects the sustainable development of the rail transit project. It includes researching and forecasting the passenger currents, compiling the rail running diagrams, controlling the train arrival and departure in the train station, ticket management, environmental protection management, energy saving measures, also includes bus and rail transit access, the guiding signs and diverting measures around stations.

3.2.5 The consumer utility

The consumer utility means the degree of consumer’s satisfaction when purchasing certain commodity. It is subjective to some extent. Therefore, it is important to insist on the people-oriented principles. It states that the urban rail transit project has influenced on the employment, income, living standards and quality of life of the residents. It analyzes the relationships among the projects, local social environment, social culture and the local residents and government. The consumer effect is described by a series of living indices, such as the degree of the passenger comfort and satisfaction, the amount of subsidies, the cost-saving of travel time, the community care etc.

3.2.6 The post evaluation of environment influence

The environment is the summation of all natural and society factors which influence on human. Therefore, the implementations of urban rail transit projects will not only affect the city’s ecological environment and natural environment, but also affect their aesthetics and social environment. The reports on the environment influence, which is compiled at the feasibility research stage, has analyzed the environmental influences caused by the project and presented the environmental protection policies and activities which mainly include reducing environmental damage on project construction and the reduction of environmental pollution in manufacturing and operating management.

3.2.7 The economic analysis

The project financial analysis itself is just analyzing the profit status. However it can not evaluate the significant influences on the national economy and social sustainable development after project completion. Therefore, the urban rail transit projects need to be evaluated on the economic effects from higher vantage. This will benefit not only the industrial structure optimization, but also be conducive to the efficient allocation of social resources. The project economic analysis usually calculates the social net benefits offered by the urban rail transit with common parameters like shadow price, shadow exchange rate, the social discount rate, shadow wage etc.

3.2.8 The public participation

The urban rail transit project is a systematical engineering. Any negligence on the project will cause huge losses to the public and leave regrets to the next generation [19]. With the rapid development of Chinese political system innovation, the public decision will be influenced more and more public suggestions. The Non-government Organization (NGO) participation model in the public decision supplements the shortcoming of the government and the market and enlarges the ways that people participate in. It is also useful to the construction of the harmonious society in China [20].

## 4 The selection of the evaluation methods

The commonly used evaluation methods are Delphi, AHP (Analytical Hierarchy Process), fuzzy evaluation and GRA method. Delphi is the main method of the qualitative prediction. It anonymously consults the expert in expert groups respectively; the suggestions will be systematically analyzed by the presider to determine the predictive value. AHP is a combination of qualitative and quantitative analysis of the decision-making methods. It is a procedure which makes the decision-making of complex system be a model and quantifiable. The decision maker can split the complex issues into several levels and some factors. The weights of different solutions can be got by comparing and calculating the factors and are provided to the selection of project solution. Fuzzy evaluation method is a mathematical method describing and manipulating fuzzy information and can be applied to uncertain and fuzzy indices. The grey correlation method evaluates the relative similarity among several factors in the system and discovers characteristics of the comparable sequences and reference sequence. The grey relational grade is the main criteria to evaluate the solutions. GRA is simple and not limited to the number of samples and the characteristics of data. It can be executed quickly by programming on computer.

According to the evaluation index system of the sustainable development of the urban rail project and the characteristics of various evaluation methods, GRA was adopted. The number of indices affecting the system is unknown and the data belong to each index are not all available in statistical documents. These are the typical characteristics of the grey system. Based on the above analysis, it is reasonable to evaluate this system using the grey system theory to remove the negative effects caused by the fuzzy mathematical analysis.

GRA analysis derives from the grey system theory and describes the development trend of a system. It finds the main factors affecting the system by comparing the reference sequence and comparable sequences. Reference sequence is the standard sequence of grey relational analysis. If the comparable sequence is , the reference sequence is ,

Then

(1)

(2)

(3)

(4)

(5)

(6)

Where is the relational coefficient of and at index, the  is the distinguishing coefficient and its value range is from 0 to 1, usually 0.5.

The grey relational grade is

(7)

Where  is the weight for the index k, and =1, the value of is determined by the knowledge, experiences and judgments of experts. The larger the grey relational grade, the higher degree of similarity between the comparable sequence and the reference sequence. The solutions can be sorted according to this grey relational grade.

## 5 The practical application

There are four built urban rail transit projects need to be evaluated for the sustainable development. The expert group assigns marks for each index based on the quantitative and qualitative analysis of evaluation indices. The final result is shown in Table 1. The comparable sequence  is generated automatically.

Table 1  The mark for each index

| Route number | Results |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Consulting planning | Overall design | Coordination of construction | Consistent operation management | Consumer utilities | Environment influence | Economy analysis | Public participation |
| a | 80 | 80 | 78 | 80 | 82 | 83 | 80 | 75 |
| b | 83 | 81 | 80 | 82 | 78 | 79 | 80 | 70 |
| c | 85 | 80 | 81 | 78 | 77 | 80 | 79 | 78 |
| d | 82 | 82 | 79 | 78 | 80 | 79 | 81 | 72 |

Step 1: get the minimum and maximum values of the sequence differences via Table 2.

Table 2  The absolute difference for each index

| Route number | Absolute difference |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Consulting planning | Overall design | Coordination of construction | Consistent operation management | Consumer utilities | Environment influence | Economy analysis | Public participation |
| a | 5 | 2 | 3 | 2 | 0 | 0 | 1 | 3 |
| b | 2 | 1 | 1 | 0 | 4 | 4 | 1 | 8 |
| c | 0 | 2 | 0 | 4 | 5 | 3 | 2 | 0 |
| d | 3 | 0 | 2 | 4 | 2 | 4 | 0 | 6 |

Step 2: when distinguishing coefficient, the calculation results of the relational coefficient  based on formula (3) are listed in Table 3.

Table 3  The calculation results of relational coefficient

| Route number | Relational coefficient |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Consulting planning | Overall design | Coordination of construction | Consistent operation management | Consumer utilities | Environment influence | Economy analysis | Public participation |
| a | 0.44 | 0.67 | 0.57 | 0.67 | 1.00 | 1.00 | 0.80 | 0.57 |
| b | 0.67 | 0.80 | 0.80 | 1.00 | 0. 50 | 0.50 | 0.80 | 0.33 |
| c | 1.00 | 0.67 | 1.00 | 0.50 | 0.44 | 0.57 | 0.67 | 1.00 |
| d | 0.57 | 1.00 | 0.67 | 0.50 | 0.67 | 0.50 | 1.00 | 0.40 |

Step 3: calculate the grey relation grade  based on formula (7). Where  is the weight of the index , and =1, the value of the  is given by expert group. The weight of the consulting planning has the highest value followed by the overall design and coordination of construction. The detailed information is as follows:

The result of the grey relation grade is presented below:

Therefore, the route c is the closest to the best sustainable development state in similarity among the four built routes.

Step 4: Results analysis. Route c is developed later than other three routes. The first seven indices affecting sustainable development in c are not much different from route a, b and d. However, public participation in route c gets higher mark than other routes. This reflects that more and more individuals and NGOs pay attention to urban rail transit project. The earlier the public participates and the more suggestions got in consulting and design phases, the more help to the decision making of the government.

However, there are many problems in the development of current Chinese NGO. The NGO development is not supported completely by the system and funds due to the lack of law and regulation. The improper internal structure affects its normal function. The non-fluent international communication and cooperation lowers the international effect of Chinese NGO [21]. Because urban rail transit is big and complex system engineering, the influence and endurance of single NGO is very limited. They participate in only the surface states such as dismantling and allocation and environmental protectection. This will make the participation process inconsistently by some activities, such as emphasizing the dismantling compensation but neglecting the comforts of the new residents, living recovery, and the adjustment of the community culture, and stressing construction and vehicle noise, but neglecting the protection of the historical relics, ecosystem balance and readjustment of the city layout.

Although urban rail transit has many advantages, there are also many environmental problems such as vibration, noise, electromagnetism radiation, earth sedimentation that occurred during the programming, designing, constructing and operating [22]. And there are also social problems such as appearing new immigration, affecting earth exploration grads of circumjacent regional, destroying historic sites. Consequently, in order to consider properly and make fewer mistakes, the urban rail transit construction project led by the government should accept the rational suggestions from all levels of society, especially the NGOs.

## 6 Conclusion

The construction of urban rail transit project has become an important method to solve the transit problems in middle and large cities and realized the sustainable development of regional economy. It is significant to research and discuss theories and methods of sustainable development of urban rail transit projects and provide reference for existing and future projects. The paper shows that GRA is an effective method to evaluate sustainable development of urban rail transit. It has the following advantages: (1) effective evaluation to consumer effect and public participation; (2) the higher the degree of the public participation, the more help to the government decision making policy and the higher performance of the sustainable development of urban rail transit project; (3) widely accepted by the public and its evaluation result is closer to the best solution.

##### References

[1]	P. A. Sun. Sustainable Urban Transportation Performance Indicators and Some Analytical Approaches [J]. Journal of Urban Planning and Development, 2002, 128(4): 184-209

[2]	Y. Zhang. Discussion on Government Functions in BOT Financing Mode of Urban Rail Transit [J]. Urban Rapid Rail Transit, 2007, 20(1): 11-13 (in Chinese)

[3]	X. M. Chen, F. J. Li. The Study on the Criterion System for Comprehensive System for Evaluation of the Urban [J]. City Planning Review, 2001, 25(10): 61-64

[4]	D. P. Wang, T. Y. Liu. On Surrounding Landscape of Urban Mass Transit [J]. Urban Mass Transit, 2006, 19(11): 19-22

[5]	D. Yan. China Urban Rail Transit in a Great Design Master’s Eyes [J]. Rail Transit, 2007, 31(2): 31-33 (in Chinese)

[6]	J.L. Deng. Control Problems of Grey System [J].Systems and Control Letters, 1982, 12 (1): 288-294

[7]	C.H. Li. Applying the Grey Prediction Model to the Global Integrated Circuit Industry [J]. Technological Forecasting and Social Change, 2003, 70(3): 563-574

[8]	H. Morita, T. Kase, Y. Tamura and S. Lwamoto. Interval Prediction of Annual Maximum Demand Using Grey Dynamic Model[J]. Electrical Power and Energy Systems, 1996, 18(4): 409-413

[9]	Y.F. Wang. Predicting Stock Price Using Fuzzy Grey Prediction System[J]. Expert Systems with Applications, 2002, 22(6): 33-39

[10]	E.C. Chirwa. Application of Grey Model GM (1, 1) to Vehicle Fatality Risk Estimation[J]. Technological Forecasting and Social Change, 2006, 73(8): 588-605

[11]	Z.P.Niu, Y.Zhu. Sustainability Evaluation of Urban Rail Transit Projects [J]. J T Tsinghua University (Sci & Tech), 2007, 47(3): 18-20

[12]	X.S.Li. Issue Concerning Sustainable Development of Urban Rail Transit [J]. Urban Transport of China, 2006, 14 (2): 8 (in Chinese)

[13]	Y.Li. Countermeasures to Urban Rail Transit [J].Railway Transport and Economy, 2007, 29(10): 54-56

[14]	L.K.Shao. Innovative Lightweight Transit Technologies for Sustainable Transportation [J]. Journal of Transport Systems Engineering and Information Technology, 2007, 17(2): 70-71

[15]	Z.Lin. Sustainable Development of Urban Rail Transit [J]. Modern Urban Transit, 2006, 12 (6): 15-16 (in Chinese)

[16]	J. Viver. Methods for Assessing Urban Public Transport Projects[J]. Public Transport International, 1998, 30(2): 24-31

[17]	S. R. Zhang, C. F. Shao. Evaluation Indices and Model of Highway Transportation Sustainable Development[J]. China Journal of Highway and Transport, 2005, 18(2): 74-78 (in Chinese)

[18]	R. G. Ma, T. Q. Li. Application of Gray Relational Decision in Highway Items Selection [J]. Journal of Chang’an University, Natural Science Edition, 2004, 24(6): 67-70

[19]	I. N. Feng. Some Problems to be Considered in the Rapid Development of Urban Rail Transit [J]. Urban Rail Transit, 2006, 19(4): 11-13 (in Chinese)

[20]	L. D. Jin. The Role of NGOs in the Process of Building a Harmonious Society [J]. Journal of Northwest University, 2006, 36(5): 29-33

[21]	M. P. Liu. The Marketization of Public Service and the Development of the Non-Governmental Organization in China [J]. Journal of sustainable development, 2007, 33(1): 121-126

[22]	Y. Liu, X. A. Gu. The Problems in Environment Impact Value of Urban Rail Transit [J]. Urban Rapid Rail Transit, 2005, 18(5): 9-11 (in Chinese)

Financed by Shanghai leading Academic Discipline Project (Project Number: B604).